Algorithm 2: Pseudo code for path planning algorithm

Algorithm 2: Route planning algorithm Procedure: Improvement of Heuristic Functions This procedure was used to modify the path planning algorithm

1: Process EscapePathPlanning 2: Import The data of fire, and graph G = (Vertices, Edges) 3: Set starting coordinates ← InitialPosition (IP) 4: Add IP to InNodeNetwork along with path sequence 5: Set termination condition (TerminateFlag) ← False, and counter k ← 0 6: While TerminateFlag is not met 7: k ← k + 1 8: Retrieve preceding node from BackupNodeNetwork with its path sequence 9: For every adjacent node 10: Obtain real-time cost ← i.e., IntegratedConstraintCost (ICC) 11: Compute the minimum ICC from current node to each target location; Meet the risk parameters on the node ＜ death threshold 12: Proceed to next node with optimal ICC 13: Append new position to route sequence 14: If next node reaches target destination TerminateFlag ← True 15: End if 16: End for 17: End while 18: End process

Network Analysis

The indoor evacuation network constitutes a fundamental prerequisite for the successful implementation of path planning algorithms, as it effectively provides a real-time mapping of indoor traversable areas relative to the physical environment. Consequently, a well-structured evacuation network can significantly enhance the accuracy parameters of the planned paths. In light of this, the BIEN proposed in this study is compared with existing indoor evacuation network construction methods in terms of key parameters to validate the effectiveness of the proposed approach.

Using the building depicted in Figure 4 as a case study, the BIEN, the Geometric Network Model (GNM) from the literature, and the indoor component of the Indoor Emergency Spatial Model (IESM) from [2, 3] were constructed respectively. Initially, these three corresponding building models were imported into the GIS environment. The nodes and edges of the three indoor evacuation network models were then quantified via feature attribute tables, with the statistical results summarized in Table 2. As observed, the number of nodes in the BIEN is at least twice that of the GNM and IESM; a similar trend is observed for the number of edges. Furthermore, the total path length of the BIEN is also at least double that of the other models. This indicates that, compared to the traditional GNM and IESM, the BIEN achieves a nearly five-time reduction in risk while the path length increases by less than double features a significantly higher topological density of navigation nodes and traversable edges, thereby offering a greater number of viable routes and enhanced pathfinding precision.

Table 2. statistical analysis of nodes and edges of indoor evacuation network structure

| Element | BIEN | GNM | IESM | Relative ratio |  | Total path length (m) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | I vs II | I vs III | I | II | III |
| Node | 1759 | 589 | 394 | 2.99 | 4.46 | 2553.49 | 1520.31 | 1298.58 |
| Edge | 2102 | 921 | 587 | 2.28 | 3.58 |  |  |  |

While the network structure shown in Figure 2 exhibits a higher density that is well-suited to human movement characteristics, the precision of the generated paths requires further verification. Consequently, a second evaluation parameter is introduced: an error analysis between the planned path and the actual path. To this end, three origin-destination pairs were selected within the building and mapped onto the three respective networks. The traditional Dijkstra was employed to perform path planning, with the results summarized in Table 3. The data indicate that the path error for the BIEN remains below 10 %, whereas the errors for the GNM and IESM generally exceed 20 % in most cases. This further demonstrates that the BIEN proposed in this study is more reliable and effective than the traditional GNM and IESM. [3].

Table 3. statistical analysis of three indoor network planning paths.

| Number | Length (m) | Relative error (%) | Length (m) |  |  | Relative error (%) |  | Reference length (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3D-TING |  | GNM | IESM |  | GNM | IESM |  |
| 1 | 79.61 | 10.0 | 55.98 |  | 58.61 | 22.6 | 18.9 | 72.31 |
| 2 | 83.58 | 8.0 | 71.75 |  | 75.45 | 21.1 | 16.9 | 90.87 |
| 3 | 120.39 | 9.5 | 101.25 |  | 99.91 | 23.9 | 24.9 | 133.08 |

Reference

[1] Zhu Z, Hu C, Chen W, Wang X. Integration of two different road network models for emergency rescue pathfinding in indoor and outdoor environments. Measurement Science and Technology 2024;35:106314. https://doi.org/10.1088/1361-6501/ad6204.

[2] Lin WY, Lin PH. Intelligent generation of indoor topology (i-GIT) for human indoor pathfinding based on IFC models and 3D GIS technology. Automation in Construction 2018;94:340–59. https://doi.org/10.1016/j.autcon.2018.07.016.

[3] Tashakkori H, Rajabifard A, Kalantari M. A new 3D indoor/outdoor spatial model for indoor emergency response facilitation. Building and Environment 2015;89:170–82. https://doi.org/10.1016/j.buildenv.2015.02.036.